Sleep Stage Classification from EEG Signals Using a Lightweight CNN

Abstract

Sleep staging classifying overnight EEG into Wake, N1, N2, N3, and REM is vital for diagnosing disorders such as insomnia and sleep apnea, but manual polysomnography scoring is slow and requires clinical expertise. This project develops a lightweight, single-channel 1D-CNN that classifies 30-second EEG epochs directly from the raw signal, targeting low-power, wearable-friendly monitoring rather than cloud-dependent clinical systems. It is trained and evaluated on the Sleep-EDF Database (PhysioNet), which provides polysomnography recordings with EEG (Fpz-Cz and Pz-Oz channels), horizontal EOG, and submental EMG signals, along with expert-annotated stage labels for each epoch following AASM/Rechtschaffen and Kales scoring conventions.

Unlike prior approaches that convert EEG into six-channel time-frequency images to achieve high accuracy [1], this design uses a single raw channel with a subject-independent train/test split for an honest measure of generalization to unseen individuals, with error analysis focused specifically on adjacent, clinically important stage confusions such as N1/REM. Raw recordings are segmented into standard 30-second epochs, normalized, and classified using a compact CNN architecture, with a night-long streaming simulation demonstrating overnight monitoring behaviour. The resulting classifier, evaluated with per-class precision, recall, F1-score, and a confusion matrix, is demonstrated through a hypnogram-style overnight visual simulation. This offers a practical, resource-constrained tool for automating overnight PSG scoring in sleep disorder diagnosis, enabling consumer wearable sleep tracking on limited hardware, supporting mental health and cognitive research through large-scale overnight data processing, and reducing dependence on costly in-lab studies for remote patient monitoring packaged with a short technical report suitable for minor project submission and viva presentation.

Keywords: EEG signal processing, sleep stage classification, lightweight CNN, single-channel wearable sensing, subject-independent evaluation, hypnogram.

Sustainable Development Goals (SDGs):

- SDG 3: Good Health and Well-Being – accessible, low-cost automated sleep monitoring supports early detection of sleep disorders and broader health outcomes linked to sleep quality.

- SDG 9: Industry, Innovation and Infrastructure – the lightweight, single-channel model design reflects efficient AI deployable on low-power wearable health devices rather than cloud-dependent clinical systems.

Group Members:

- Athira S (B23CS2122)

- Avin Joy (B23CS2123)

- Divina Ann Shaji (B23CS2126)

- V. Anantha Krishnan (B23CS2163)

Guide: Mrs. Divya R, Assistant Professor, Department of Electronics and Communication

References

[1] I. S. Masad, A. Alqudah, and S. Qazan, “Automatic Classification of Sleep Stages Using EEG Signals and Convolutional Neural Networks,” PLOS ONE, vol. 19, no. 1, e0297582, 2024.

[2] C. Yang, B. Li, Y. Li, Y. He, and Y. Zhang, “LWSleepNet: A Lightweight Attention-Based Deep Learning Model for Sleep Staging with Single-Channel EEG,” Digital Health, vol. 9, 2023.

[3] S. Zhang and X. Wang, “EEG-Based Sleep Stage Classification for Entrepreneurial Students: A Lightweight and Efficient Deep Learning Approach,” Journal of Mechanics in Medicine and Biology, 2025.

[4] H. Zhu et al., “A Deep Learning Method Approach for Sleep Stage Classification with EEG Spectrogram,” International Journal of Environmental Research and Public Health, vol. 19, no. 10, p. 6322, 2022.

[5] “Employing a Convolutional Neural Network to Classify Sleep Stages from EEG Signals Using Feature Reduction Techniques,” Algorithms, vol. 17, no. 6, p. 229, 2024.